ECA10

MICROPROCESSORS AND

MICROCONTROLLERS

Name: Yarra Pawan

Reg No: 192424294

RECORD

INDEX

| S. No | Name of the Experiment | Page No. |
| --- | --- | --- |
| 8085 Microprocessor |  |  |
| 1. | Arithmetic Operations using 8085 Microprocessor |  |
| 2. | Ascending and Descending Order of an Unsorted Array using 8085 |  |
| 3. | ASCII to HEX (Binary) Conversion using 8085 Microprocessor |  |
| 4. | Program transferring a block of data between memory locations using 8085 |  |
| 5. | Perform Masking (AND), Setting (OR) of lower nibbles and One’s and Two’s complement on given data using 8085 Microprocessor |  |

2

1. Arithmetic Operations in 8085 Microprocessors

a. 8-BIT ADDITION WITH CARRY USING DIRECT ADDRESSING AIM:

To write an assembly language program to add two numbers of 8-bit data stored in memory  locations 4200H and 4201H and store the result in 4202H and 4203H with carry using direct  addressing.

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Load the first data from memory to the accumulator and move it to B register. 2. Load the second data from memory to the accumulator.

3. Add the content of B – register to accumulator

4. If Carry flag = 0 then jump to step 6

5. Increment C register to count the carry

6. Store the sum in memory.

7. Move the carry to accumulator and store in memory.

8. Stop.

PROGRAM TO ADD TWO 8-BIT DATA

| Memory address | Label | Instruction | Opcode | Comments |
| --- | --- | --- | --- | --- |
| 4100 |  | LDA 4200H |  | Get 1st data in A and save in B. |
| 4103 |  | MOV B, A |  |  |
| 4104 |  | LDA 4201H |  | Get 2nd data in A-register |
| 4107 |  | ADD B |  | Get the sum in A register |
| 4108 |  | JNC SKIP |  | If CY=0 Then skip next step |
| 410B |  | INR C |  | Increment C register to count the carry |
| 410C | SKIP | STA 4202H |  | Store the sum in memory |
| 410F |  | MOV A,C |  | Move the carry to accumulator and store in memory |
| 4110 |  | STA 4203H |  |  |
| 4113 |  | HLT |  | Stop the Execution |

| Input |  | Output |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 4200 | CF | 4202 | 6C (Sum) |
| 4201 | 9D | 4203 | 01 (Carry) |

RESULT:

Thus, an assembly language program for addition of two 8-bit numbers with carry was written, executed  and Verified the Result successfully using 8085 kit.

3

b. 8-BIT ADDITION USING INDIRECT ADDRESSING

AIM:

To write an assembly language program to add two numbers of 8-bit data stored in memory  locations 4200H and 4201H and store the result in 4202H and 4203H using indirect  addressing modes

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Load the first data from memory to the accumulator.

2. Add the content of Memory to accumulator

3. If Carry flag = 0 then jump to step 5

4. Increment C register to count the carry

5. Store the sum in memory.

6. Store the carry in memory from C-register.

7. Stop.

PROGRAM TO ADD TWO 8-BIT DATA

| Memory address | Label | Instruction | Opcode | Comments |
| --- | --- | --- | --- | --- |
| 4100 |  | LXI H,4200H |  | Load the HL with data address |
| 4103 |  | MVI C,00 |  | Clear C-Reg to count carry |
| 4105 |  | MOV A, M |  | Get 1st data in A |
| 4106 |  | INX H |  |  |
| 4107 |  | ADD M |  | Add reg-A with Memory and Get the sum in A register |
| 4108 |  | JNC SKIP |  | If CY=0, Skip next step |
| 410B |  | INR C |  | Increment C-Reg |
| 410C | SKIP | INX H |  |  |
| 410D |  | MOV M, A |  | Store the sum in memory |
| 410E |  | INX H |  |  |
| 410F |  | MOV M, C |  | Store the Carry in memory |
| 4110 |  | HLT |  | Stop the Execution |

SAMPLE DATA:

| Input |  | Output |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 4200 | 2F | 4202 | 18 (Sum) |
| 4201 | E9 | 4203 | 01 (Carry) |

RESULT:

Thus, an assembly language program for addition of two 8-bit numbers with carry was written, executed  and Verified the Result successfully using 8085 kit.

4

c. 8-BIT SUBTRACTION WITHOUT BORROW

AIM:

To write an assembly language program to subtract two numbers of 8-bit data stored in memory  4200H and 4201H. Store the magnitude of the result in 4202H using 8085 Microprocessor.

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Load the subtrahend (the data to be subtracted) from memory to accumulator and move  if to B-register.

2. Load the minuend from memory to accumulator.

3. Subtract the content of B-register (subtrahend) from the content of accumulator  (minuend).

4. Store the difference in memory.

5. Stop.

PROGRAM TO SUBTRACT TWO 8-BIT DATA

| Memory address | Label | Instruction | Opcode | Comments |
| --- | --- | --- | --- | --- |
| 4100 |  | LDA 4201H |  | ; Get the subtrahend in B register. |
| 4103 |  | MOV B,A |  |  |
| 4104 |  | LDA 4200H |  | ;Get the minuend in A register |
| 4107 |  | SUB B |  | ; Get the difference in A register. |
| 4108 |  | STA 4202H |  | Store the result in memory |
| 410B |  | HLT |  | Stop the Execution |

Sample data

| Address | Input Data | Address | Output Data |
| --- | --- | --- | --- |
| 4200 | D5 | 4202 | 8B (Difference) |
| 4201 | 4A |  |  |

RESULT:

Thus, an assembly language program for subtraction of two 8-bit numbers without borrow was written,  executed and Verified the Result successfully using 8085 kit.

5

d. 8-BIT SUBTRACTION WITH BORROW USING DIRECT  ADDRESSING

AIM:

To write an assembly language program to subtract tow numbers of 8-bit data stored in  memory locations 4200H and 4201H and store the result in 4202H and 4203H with borrow  using direct addressing.

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Load the second data from memory to the accumulator and move it to the B register. 2. Load the first data from memory to the accumulator.

3. Subtract the content of B – register from accumulator

4. If Carry flag = 0 then jump to step 5 & 6

5. Increment C register to count the borrow

6. Take two’s complement of the difference

7. Store the Difference in memory.

8. Move the borrow to accumulator and store in memory.

9. Stop.

PROGRAM TO SUBTRACT TWO 8-BIT DATA

| Memory address | Label | Instruction | Opcode | Comments |
| --- | --- | --- | --- | --- |
| 4100 |  | LDA 4201H |  | Get 2nd data in A and save in B. |
| 4103 |  | MOV B, A |  |  |
| 4104 |  | LDA 4200H |  | Get 1st data in A-register |
| 4107 |  | SUB B |  | Subtract B-Reg from A register |
| 4108 |  | JNC SKIP |  | If CY=0 Then skip next two steps |
| 410B |  | INR C |  | Increment C register to count the carry |
| 410C |  | CMA |  | Take two’s complement of difference |
| 410D |  | INR A |  |  |
| 410E | SKIP | STA 4202H |  | Store the Difference in memory |
| 4111 |  | MOV A,C |  | Move the Borrow to accumulator and store in memory |
| 4112 |  | STA 4203H |  |  |
| 4115 |  | HLT |  | Stop the Execution |

6

SAMPLE DATA:

| Input |  | Output |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 4200 | CF | 4202 | 0E (Sum) |
| 4201 | DD | 4203 | 01 (Borrow) |

RESULT:

Thus, an assembly language program for subtraction of two 8-bit numbers with borrow was written,  executed and Verified the Result successfully using 8085 kit.

7

e. 8-BIT MULTIPLICATION OPERATIONS USING 8085  MICROPROCESSOR

AIM:

To write an assembly language program to multiply two numbers of 8-bit data stored in  memory 4200H and 4201H and store the product in 4202H and 4203H.

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

● Load the first data in ACC and move to E.

● Load the second data ACC and move to B (count)

● Clear HL pair (Initial sum)

● Clear D for overflow (carry)

● Add the content of DE to HL

● Decrement the count.

● Check whether the count has reached zero.

● Check the zero flag. If ZF = 0, repeat addition or If ZF = 1, go to next step ● Store the content of HL in memory. (Least significant 16 bits of the product)  ● Stop.

PROGRAM TO MULTIPLY TWO NUMBERS OF 8-BIT DATA

Memory

address Label Instruction Opcode Comments

| 4100 |  | LDA 4200H |  | ;Get 1 st data in A |
| --- | --- | --- | --- | --- |
| 4103 |  | MOV E, A |  | ;Save 1st data in E |
| 4104 |  | LDA 4201H |  | ;Get 2nd data in A |
| 4107 |  | MOV B, A |  | ;save 2nd data in B |
| 4108 |  | LXI H,0000H |  | ;Clear HL pair(initial sum=0) |
| 410B |  | MVI D,00H |  | ;Clear E for accounting overflow. |
| 410D | NEXT: | DAD D |  | ;Add the content of DE to sum(HL) |
| 410E |  | DCR B |  | Decrement data 2 for every addition |
| 410F |  | JNZ NEXT |  | ;Repeat Addition until count is zero. |
| 4112 |  | SHLD 4202H |  | ;Store the product in memory |
| 4115 |  | HLT |  | Stop the Execution |

Sample data

| Address | Input Data | Address | Output Data |
| --- | --- | --- | --- |
| 4200 | 6D (Data-1) | 4202 | 26 (Lower byte of product) |
| 4201 | FE (Data-2) | 4203 | 6C (Higher byte of product) |

RESULT:

Thus, an assembly language program to multiply two numbers of 8-bit data was written,  executed and Verified the Result successfully using 8085 kit.

8

f. 8-BIT DIVISION OPERATIONS USING 8085

MICROPROCESSOR

AIM:

To write an ALP to perform division of two 8 bit numbers Stored in memory location  4200H, 4201 H and Store the remainder in 4202H and the quotient in 4203H.

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

● Load the divisor in accumulator and move if to B-register

● Load the dividend in the accumulator.

● Clear C-register to account for quotient

● Check whether divisor is less than dividend

● If divisor is less than dividend, go to step 8, otherwise go to next step ● Subtract the content of B-register (quotient)

● Increment the content of C-register (quotient)

● Go to step 4

● Store the content of the accumulator (remainder) in memory.

● Move the content of C-register (quotient) to accumulator and store in memory ● Stop.

PROGRAM TO DIVIDE TWO NUMBERS OF 8-BIT DATA

| Memory address Label Instruction Comments |  |  |  |
| --- | --- | --- | --- |
| 4100 |  | LDA 4201H |  |
| 4103 |  | MOV B,A | ;Get the divisor in B register |
| 4104 |  | LDA 4200H | ;Get the dividend in A register |
| 4107 |  | MVI C,00H | ;Clear C register for quotient |
| 4109 | AGAIN: | CMP B |  |
| 410A |  | JC STORE | ;If divisor is less than dividend go to store |
| 410D |  | SUB B | ;Subtract divisor from dividend. Increment |
| 410E |  | INR C | ;quotient by one for each subtraction. |
| 410F |  | JMP AGAIN |  |
| 4112 | STORE: | STA 4203H | ;Store the remainder in memory |
| 4115 |  | MOV A,C |  |
| 4116 |  | STA 4202H | ;Stare the quotient in memory |
| 4119 |  | HLT | Stop the Execution |

Sample data

| Address | Input Data | Address | Output Data |
| --- | --- | --- | --- |
| 4200 | 9F (Dividend) | 4202 | 0F (Quotient) |
| 4201 | 0A (Divisor) | 4203 | 09 (Remainder) |

RESULT:

Thus, an assembly language program to Divide two numbers of 8-bit data was written, executed  and Verified the Result successfully using 8085 kit.

Test Cases: Arithmetic Operations of 8085 Microprocessor

Test Case Question 1: These test cases are for the arithmetic operations such as addition and subtraction. Each test case includes the initial values of registers and memory locations, the operation to be performed, and the expected results. ● Initial Register Values: Accumulator (A) = 35H B Register (B) = 25H Operation: What will be the value in the accumulator (A) and the carry flag (CY) after executing the instruction ADD B? Output: ● The value in the accumulator (A) after the operation will be 5AH. ● The carry flag (CY) will be 0. ● Initial Register Values: Accumulator (A) = 6EH B Register (B) = 3CH Operation: What will be the value in the accumulator (A) and the carry flag (CY) after executing the instruction SUB B? Output: 4. The value in the accumulator (A) after the operation will be 32H. 5. The carry flag (CY) will be 0 (no borrow). Test Case Question 2: These test cases are for the arithmetic operations such as multiplication, and division. Each test case includes the initial values of registers and memory locations, the operation to be performed, and the expected results. (i) Initial Register Values: Accumulator (A) = 14H B Register (B) = 0AH Operation: What will be the values in the accumulator (A) and the B Register (B) after executing the instruction MUL B? Output: 1. The value in the accumulator (A) after the operation will be C8H. 2. The value in the B register (B) will remain 0AH, as typically multiplication affects only the accumulator. (ii) Initial Register Values: Accumulator (A) = 63H (Dividend) B Register (B) = 09H (Divisor) Operation: What will be the values in the accumulator (A) and the B Register (B) after executing the instruction DIV B? Output: ● The value in the accumulator (A) after the operation will be 0BH (quotient). ● The value in the B register (B) will be 00H (remainder).

10

2a.SORT AN ARRAY OF DATA IN ASCENDING ORDER

AIM:

To write an assembly language program to sort an array of data in ascending order. The array  is stored in memory starting from 4200H. The first element of the array gives the count value  for the number of elements in the array.

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

● Load the count value from memory to A-register and save it in B-register ● Decrement B-register (B is a count for (N-1) repetitions)

● Set HL pair as data address pointer

● Set C-register as counter for (N-1) comparisons.

● Load a data of the array in accumulator using the data address pointer ● Increment the HL pair (data address pointer)

● Compare the data pointed by HL wait accumulator

● If carry flag is set (If the content of accumulator is smaller than memory) then go to  step 10, otherwise go to next step

● Exchange the content of memory pointed by HL and the accumulator ● Decrement C-register. If zero flag is reset go to step 6 otherwise go to next step ● Decrement B-register. If zero flag is reset go the step 3 otherwise go to next step ● Stop.

PROGRAM TO SORT AN ARRAY OF DATA IN ASCENDING ORDER

| Memory address | Label | Instruction |  | Opcode | Comments |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | LDA | 4200H |  | ;Load the count value |
| 4103 |  | MOV | B,A |  | ;Set counter for (N-1) repetitions |
| 4104 |  | DCR | B |  | ;of (N-1) comparisons |
| 4105 | LOOP 2 | LXI | H,4200H |  | ;Set pointer for array |
| 4108 |  | MOV | C,M |  | ;Set count for (N-1) comparisons |
| 4109 |  | DCR | C |  |  |
| 410A |  | INX | H |  | ;Increment pointer |
| 410B | LOOP 1 | MOV | A,M |  | ;Get one data of array in A |
| 410C |  | INX | H |  |  |
| 410D |  | CMP | M |  | ;Compare next data with A register |
| 410E |  | JC | AHEAD |  | ;If content of A is less than memory then go to AHEAD |
| 4111 |  | MOV | D,M |  | ;If the content of A is greater than |
| 4112 |  | MOV | M,A |  | ;then content of memory |

11

| 4113 |  | DCX | H |  | ;pointed by HL and previous location |
| --- | --- | --- | --- | --- | --- |
| 4114 |  | MOV | M,D |  |  |
| 4115 |  | INX | H |  |  |
| 4116 | AHEAD | DCR | C |  | ;Repeat comparisons until C count is zero |
| 4117 |  | JNZ | LOOP l |  |  |
| 411A |  | DCR | B |  | ;Repeat until B count is zero |
| 411B |  | JNZ | LOOP 2 |  |  |
| 411E |  | HLT |  |  | Stop the Execution |

Sample Data

| Address | Data Array (Before sorting) | Address | Data Array (After sorting) |
| --- | --- | --- | --- |
| 4200 | 07 (Count) | 4200 | 07 (Count) |
| 4201 | AB (Data -1) | 4201 | 34 (Data -1) |
| 4202 | 92 (Data -2) | 4202 | 4F (Data -2) |
| 4203 | 84 (Data -3) | 4203 | 69 (Data -3) |
| 4204 | 4F (Data -4) | 4204 | 84 (Data -4) |
| 4205 | 69 (Data -5) | 4205 | 92 (Data -5) |
| 4206 | F2 (Data -6) | 4206 | AB (Data -6) |
| 4207 | 34 (Data -7) | 4207 | F2 (Data -7) |

RESULT:

Thus, an assembly language program for sorting in Ascending order of an unsorted array of given 8-bit  numbers was written, executed and Verified the Result successfully using 8085 kit

2b. SORT AN ARRAY OF DATA IN DESCENDING ORDER

AIM:

To write an assembly language program to sort the array of data in descending order. The array  is stored in memory stored in memory starting from 4200H. The first element of the array  gives the count value for the number of elements in the array.

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

● Load the count value from memory to A-register and save it in B-register ● Decrement B-register (B is a count for (N-1) repetitions)

● Set HL pair as data address pointer

● Set C-register as counter for (N-1) comparisons.

● Load a data of the array in accumulator using the data address pointer

● Increment the HL pair (data address pointer)

● Compare the data pointed by HL wait accumulator

● If carry flag is reset (If content of accumulator is larger than memory) then go to step  10,

otherwise go to next step

● Exchange the content of memory pointed by HL and the accumulator

● Decrement C-register. If zero flag is reset go to step 6 otherwise go to next step ● Decrement B-register. If zero flag is reset go the step 3 otherwise go to next step ● Stop.

The algorithm is the same as the algorithm of example program 15 except step 8. Step 8: If carry flag is reset (If content of accumulator is larger than memory) then go to step  10, otherwise go to next step

PROGRAM TO SORT AN ARRAY OF DATA IN DESCENDING ORDER

| Memory address | Label | Instruction |  | Opcode | Comments |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | LDA | 4200H |  | ;Load the count value |
| 4103 |  | MOV | B,A |  | ;Set counter for (N-1) repetitions |
| 4104 |  | DCR | B |  | ;of (N-1) comparisons |
| 4105 | LOOP 2 | LXI | H,4200H |  | ;Set pointer for array |
| 4108 |  | MOV | C,M |  | ;Set count for (N-1) comparisons |
| 4109 |  | DCR | C |  |  |
| 410A |  | INX | H |  | ;Increment pointer |
| 410B | LOOP 1 | MOV | A,M |  | ;Get one data of array in A |
| 410C |  | INX | H |  |  |
| 410D |  | CMP | M |  | ;Compare next data with A register |
| 410E |  | JNC | AHEAD |  | ;If content of A is less than memory then go to AHEAD |

13

| 4111 |  | MOV | D,M |  | ;If the content of A is greater than |
| --- | --- | --- | --- | --- | --- |
| 4112 |  | MOV | M,A |  | ;then content of memory |
| 4113 |  | DCX | H |  | ;pointed by HL and previous location |
| 4114 |  | MOV | M,D |  |  |
| 4115 |  | INX | H |  |  |
| 4116 | AHEAD | DCR | C |  | ;Repeat comparisons until C count is zero |
| 4117 |  | JNZ | LOOP l |  |  |
| 411A |  | DCR | B |  | ;Repeat until B count is zero |
| 411B |  | JNZ | LOOP 2 |  |  |
| 411E |  | HLT |  |  | Stop the Execution |

SAMPLE DATA:

| Address | Data Array (Before sorting) | Address | Data Array (After sorting) |
| --- | --- | --- | --- |
| 4200 | 07 (Count) | 4200 | 07 (Count) |
| 4201 | AB (Data -1) | 4201 | F2 (Data -7) |
| 4202 | 92 (Data -2) | 4202 | AB (Data -6) |
| 4203 | 84 (Data -3) | 4203 | 92 (Data -5) |
| 4204 | 4F (Data -4) | 4204 | 84 (Data -4) |
| 4205 | 69 (Data -5) | 4205 | 69 (Data -3) |
| 4206 | F2 (Data -6) | 4206 | 4F (Data -2) |
| 4207 | 34 (Data -7) | 4207 | 34 (Data -1) |

RESULT:

Thus, an assembly language program for sorting in descending order of an unsorted array of given 8- bit numbers was written, executed and Verified the Result successfully using 8085 kit.

14

Test Cases: Array Operations using 8085 Microprocessor

Test Case Question 1: a) i)LDA 2050 MOV B, A LDA 2051 CMP JNC ** MOV A, B ** STA 2052 In the above Instructions explain the function of compare instruction and what will be the output when you are giving “n” 8- bit numbers. a ii) Given two 8-bit numbers stored in memory locations 3000H and 3001H as 56H and A3H respectively, write an 8085 assembly program to find the largest number and store it in memory location 3002H output : The value stored at 3002H should be A3H. Test Case Question 2: i)Test Case Question: Consider an array with 7 elements 23H, 0AH, 67H, 12H, 4BH, 3CH, 19H stored starting at 2001H, with the size 07H stored at 2000H. Write an 8085 assembly program to sort this array in ascending order. Output: The sorted array at 2001H should be 0AH, 12H, 19H, 23H, 3CH, 4BH, 67H. ii)Test Case Question:Output: Given an array stored at 2001H with elements 7FH, 80H, 0AH, FFH, and the size 04H stored at 2000H, write an 8085 assembly program to sort this array in ascending order, considering 2's complement representation. Output: The sorted array at 2001H should be 80H, FFH, 0AH, 7FH. Test Case Question 3: i ) Test Case Question:Given an array stored in memory starting at location 2001H with 4 elements: 08H, 04H, 0AH, 02H, and the size of the array (04H) stored at 2000H, write an 8085 assembly program to sort this array in descending order. Output : After execution, the array at 2001H should be sorted as 0AH, 08H, 04H, 02H. ii) Test Case Question: Given an array stored at 2001H with elements 0FH, F8H, 04H, FFH, and the size 04H stored at 2000H, write an 8085 assembly program to sort this array in descending order, considering 2's complement representation Output :The sorted array at 2001H should be 0FH, 04H, F8H, FFH.

15

1. ASCII CODE TO HEX CODE CONVERSION

AIM:

To write an assembly language program to convert an array of ASCII codes to corresponding  binary (Hex) value in 8085 Microprocessor

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Get the ASCII data in A register from 4200H

2. Subtract 30H from A-register

3. Compare the content of A-register with 0AH

4. If CY = 1 go to step 6. If CY = 0, go to next step

5. Subtract 07H from A-register

6. Store the HEX into 4202H

7. Stop the program

PROGRAM TO CONVERT ASCII CODE TO BINARY VALUE

| Memory address | Label | Instruction |  | OP Code | Comments |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | LDA | 4200 |  | Get the ASCII date to A register |
| 4103 |  | SUI | 30H |  | ;Subtract 30h from the data |
| 4105 |  | CPI | 0AH |  | Compare the result with 0A |
| 4107 |  | JC | STORE |  | ;If CY = 1, Store the result |
| 410A |  | SUI | 07H |  | ;Else then subtract 07H |
| 410C | STORE | STA | 4201 |  | Store the result |
| 410F |  | HLT |  |  | Stop the program |

SAMPLE DATA:

| ASCII Input |  | Hex Output |  |
| --- | --- | --- | --- |
| 4200 | 40 | 4201 | 0A |

RESULT:

Thus, an assembly language program for converting ASCII to HEX of given 8-bit number was written,  executed and Verified the Result successfully using 8085 kit.

16

Test Cases: Code Conversion using 8085 Microprocessor

Test Case Question : Convert an ASCII character ('0' to '9') stored in memory into its decimal equivalent and store the result in another memory location. Test Case Scenarios and Outputs: Test Case 1: 1. Input (ASCII): The memory location 2500H contains 0x35 (ASCII for '5'). • The ASCII value (0x35) corresponds to '5'. • Subtracting 0x30 from 0x35 gives 0x05, which is the decimal value 5. • The result (0x05) will be stored at memory location 2501H. 2. Output: • 2501H: 0x05 (Decimal 5) Test Case 2: i) Input (ASCII): The memory location 2500H contains 0x31 (ASCII for '1'). • The ASCII value (0x31) corresponds to '1'. • Subtracting 0x30 from 0x31 gives 0x01, which is the decimal value 1. • The result (0x01) will be stored at memory location 2501H. ii) Output: • 2501H: 0x01 (Decimal 1) Test Case 3: ● Input (Invalid ASCII): The memory location 2500H contains 0x41 (ASCII for 'A'). : ○ Since the value is outside the range of ASCII digits (0x30–0x39), the program will jump to the INVALID label and halt without modifying memory. ● Output: ○ The program halts without storing anything in 2501H. Test Case 4: ● Input (ASCII): The memory location 2500H contains 0x39 (ASCII for '9'). ○ The ASCII value (0x39) corresponds to '9'. ○ Subtracting 0x30 from 0x39 gives 0x09, which is the decimal value 9. ○ The result (0x09) will be stored at memory location 2501H. ● Output: ○ 2501H: 0x09 (Decimal 9)

17

2. TRANSFERRING A BLOCK OF DATA BETWEEN MEMORY LOCATIONS  AIM:

To write an assembly language program to transfer a block of data from source to destination  in 8085 Microprocessor

APPARATUS REQUIRED:

1. 8085 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Initialize BC pair with 4200H

2. Initialize DE pair with 4300H

3. Initialize H Reg with number of bytes to be transferred

4. Load a Byte from source address and Store the same on Destination address 5. Check all the data bytes transferred if no repeat step 4

6. Stop the program

| Memory address | Label | Instruction | Comments |
| --- | --- | --- | --- |
| 4100 |  | LXI B,4200H | Initialize BC pair with 4200H |
| 4103 |  | LXI D,4300H | Initialize DE pair with 4300H |
| 4106 |  | MVI H,07 | Initialize H Reg with number of bytes to be transferred |
| 4108 | NEXT | LDAX B | Load a Byte from source address |
| 4109 |  | STAX D | Store the same on Destination address |
| 410A |  | INX B | Repeat the above steps to transfer the entire string |
| 410B |  | INX D |  |
| 410C |  | DCR H |  |
| 410D |  | JNZ NEXT |  |
| 4110 |  | HLT | Stop the process |

SAMPLE DATA:

| Address | Source Array | Address | Destination Array |
| --- | --- | --- | --- |
| 4200 | 11 | 4300. | 11 |
| 4201 | 22 | 4301. | 22 |
| 4202 | 33 | 4302. | 33 |
| 4203 | AA | 4303. | AA |
| 4204 | BB | 4304. | BB |
| 4205 | CC | 4305. | CC |
| 4206 | DD | 4306. | DD |
| 4207 | EE | 4307. | EE |

RESULT:

Thus, an assembly language program for Transferring of given 8-bit numbers was written, executed and  Verified the Result successfully using 8085 kit.

18

5a. MASKING AND SETTING OF LOWER NIBBLES ON GIVEN DATA

AIM:

To write and execute an assembly language program for performing Masking, Setting, One’s and  Two’s Complement of given data of 8-bit numbers using 8085 Microprocessor.

APPARATUS REQUIRED:

8085 microprocessor kit 1

Power card 1

Keyboard 1

MASKING OF BITS ALGORITHM:

1. Load the Data in A-register.

2. Logically AND the content of A with 0FH.

3. Store the result in memory location.

4. Stop the program

PROGRAM:

iii) By using 8086 kit:

| ADDRESS | LABEL | MNEMONICS | OPCODE | COMMENTS |
| --- | --- | --- | --- | --- |
| 4100 |  | LDA 4200 |  | Load A-register with Data |
| 4103 |  | ANI, 0FH |  | AND the content of A with 0FH |
| 4105 |  | STA 4201 |  | Store the Result |
| 4108 |  | HLT |  | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 4200H | 4A | 4201H | 0A |

19

SETTING OF BITS ALGORITHM:

1. Load the Data in A-register.

2. Logically ORI the content of A with 0FH..

3. Store the result in memory location.

4. Stop the program

PROGRAM:

iv) By using 8086 kit:

| ADDRESS | OPCODE | LABEL | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 4100 |  |  | LDA 4200 | Load A-register with Data |
| 4103 |  |  | ORI, 0FH | OR the content of A with 0FH |
| 4105 |  |  | STA 4201 | Store the Result |
| 4108 |  |  | HLT | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 4200H | C5 | 4201H | CF |

RESULT:

Thus, an assembly language program for performing logical Masking and Setting of bits were  executed using 8085 kit.

5b.ONE’S AND TWO’S COMPLEMENT

AIM:

To write and execute an assembly language program for performing One’s and Two’s Complement of  given 8-bit numbers using 8085 Microprocessor.

APPARATUS:

8085 microprocessor kit 1

Power card 1

Keyboard 1

MASKING OF BITS

ALGORITHM:

Load the Data in A-register.

Logically NOT the content of A.

Store the One’s complement in memory location.

Increment the content of A.

Store the Two’s complement in memory location.

Stop the program

PROGRAM:

v) By using 8086 kit:

| ADDRESS | OPCODE LABEL |  | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 4100 |  |  | LDA 4200 | Load AL-register with 1st Data |
| 4103 |  |  | CMA | NOT the content of AX |
| 4104 |  |  | STA 4201 | Store the One’s complement in memory location. |
| 4107 |  |  | INR A | Increment the content of AX. |
| 4108 |  |  | STA 4202 | Store the Two’s complement in memory location |
| 410B |  |  | HLT | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 4200H | AB | 4201H | 54 |
|  |  | 4202H | 55 |

RESULT:

Thus, an assembly language program for performing One’s and Two’s Complement of bits were  executed using 8085 kit.

21

Test Cases: Logical operation using 8085 Microprocessor

Test case question 1: Load the accumulator with a random value like 0xB5 and apply ANI 0F.What is the accumulator value when a random bit pattern like 0xB5 is masked with ANI 0F? 10110101 AND 00001111 ------------ 00000101 OUTPUT: After executing ANI 0x0F, the accumulator will contain 0x05. Test case question 2: Load the accumulator with 0x1F and apply the ANI instruction with the immediate data 0x0F.Does the ANI instruction correctly preserve the lower nibble when applied to 0x1F with ANI 0F? 00011111 AND 00001111 ------------ 00001111 OUTPUT: After executing ANI 0x0F, the accumulator will contain 0x0F. The lower nibble is correctly preserved. Test case question 3: Load the accumulator with 0x00 and apply the ORI instruction with the immediate data 0x0F.What is the result in the accumulator when ORI 0F is executed on 0x00? 00000000 OR 00001111 ------------ 00001111 OUTPUT:After executing ORI 0x0F, the accumulator will contain 0x0F. Test case question 4:Load the accumulator with 0xFF and perform the one’s complement.What happens to 0xFF after performing the one’s complement? ~11111111 --------- 00000000 OUTPUT:After performing the one’s complement on 0xFF, the accumulator will contain 0x00. Test case question 5:Load the accumulator with 0x80 (representing -128 in signed 8-bit) and perform the one’s complement.How does the one’s complement work when the accumulator contains a signed negative number like 0x80? ~10000000 --------- 01111111 OUTPUT: After performing the one’s complement on 0x80, the accumulator will contain 0x7F (which is 127 in signed 8-bit).

22