ECA10

MICROPROCESSORS AND

MICROCONTROLLERS

Name: Yarra Pawan

Reg No: 192424294

MASTER RECORD

With

Test cases

1

INDEX

| S. No | Name of the Experiment | Page No. |
| --- | --- | --- |
| 8086 Microprocessor |  |  |
| 6. | 16-bit Addition using indirect addressing with carry in 8086 Microprocessor |  |
| 7. | 16-bit Subtraction with borrow in 8086 Microprocessor |  |
| 8. | 16-bit Multiplication operation using 8086 Microprocessor |  |
| 9. | 16-bit Division operation using 8086 Microprocessor |  |
| 10. | Logical operations in 8086 Microprocessor [a] Masking of bits using AND operation. [b] Setting of bits using OR operation. [c] Exclusive-OR operation using 8086 Microprocessor |  |
| 11. | 1’s and 2's complement of a 16-bit number using 8086 Microprocessor |  |
| 12. | String Operation using 8086 Microprocessors: Move a block of data from source to destination |  |
| 13. | Sum of N Numbers in a word array using 8086 Microprocessor |  |
| 14. | Factorial of number using 8086 microprocessor |  |
| 15. | Square of number using 8086 microprocessor |  |

22

6. ADDITION OF 16 BIT NUMBERS WITH CARRY

AIM:

To write and execute an assembly language program to add two 16-bit unsigned numbers with carry  in 8086 kit.

APPARATUS:

1. 8086 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Load the First Data in AX-register.

2. Load the First Data in BX-register.

2. Add the two data and get the sum in AX-register.

3. If C=0 then skip next step.

4. Increment CX Reg for carry

5. Store the sum in memory locations.

6. Store the Carry in memory location.

7. Stop the program.

PROGRAMM

| ADDRESSLABEL |  | MNEMONICS | OPCODE | COMMENT |
| --- | --- | --- | --- | --- |
| 1100 |  | MOV CX,0000H |  | Initialize counter CX |
| 1104 |  | MOV AX,[1200] |  | Get the first data in AX register. |
| 1108 |  | MOV BX,[1202] |  | Get the second data in BX register. |
| 110C |  | ADD AX,BX |  | Add the contents of both the register AX & BX |
| 110E |  | JNC L1 |  | Check for carry |
| 1110 |  | INC CX |  | If carry exists, increment the CX |
| 1111 | LI | MOV [1206],CX |  | Store the carry |
| 1113 |  | MOV [1204],AX |  | Store the sum |
| 1117 |  | HLT |  | Stop the program |

OUTPUT FOR ADDITION:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1200 | ABCD | 1204 | 9ADF |
| 1202 | EF12 | 1206 | 0001 |

RESULT

Thus, an assembly language program for addition with carry of given 16-bit numbers was written,  executed and Verified the Result successfully using 8086 kit

.

23

7.SUBTRACTION OF 16 BIT NUMBERS WITH BORROW

AIM

To write and execute an assembly language program to subtract two 16-bit unsigned numbers with  borrow in 8086 kit.

APPARATUS:

1. 8086 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Load the second data from memory to the accumulator and move it to B register. 2. Load the first data from memory to the accumulator.

3. Subtract the content of B – register from accumulator

4. If Carry flag = 0 then jump to step 5 & 6

5. Increment C register to count the borrow

6. Take two’s complement of the difference

7. Store the Difference in memory.

8. Move the borrow to accumulator and store in memory.

9. Stop.

PROGRAMM

| ADDRESS | LABEL | MNEMONICS | OPCODE | COMMENT |
| --- | --- | --- | --- | --- |
| 1100 |  | MOV CX,0000H |  | Initialize counter CX |
| 1104 |  | MOV AX,[1300] |  | Get the first data in AX register |
| 1108 |  | MOV BX,[1302] |  | Get the second data in BX register. |
| 110C |  | SUB AX,BX |  | Subtract the contents of both the register AX & BX |
| 110E |  | JNC SKIP |  | Check the Borrow. |
| 1110 |  | INC CX |  | If carry exists, increment the CX |
| 1111 |  | NEG AX |  | Take two’s complement of the difference |
| 1113 | SKIP | MOV [1306],CX |  | Store the Borrow. |
| 1117 |  | MOV [1304],AX |  | Store the difference. |
| 111A |  | HLT |  | Stop the program |

OUTPUT FOR SUBTRACTION:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1200 | ABCD | 1204 | 4345 |
| 1202 | EF12 | 1206 | 0001 |

RESULT

Thus, an assembly language program for subtraction with borrow of given 16-bit numbers was written,  executed and Verified the Result successfully using 8086 kit.

24

8. MULTIPLICATION OF 16 BIT NUMBERS

AIM

To write and execute an assembly language program to Multiply two 16-bit unsigned numbers in  8086 kit.

APPARATUS:

8086 microprocessor kit 1

Power card 1

Keyboard 1

ALGORITHM:

1. Load the multiplier from memory to accumulator.

2. Load the Multiplicand from memory to BX Reg .

3. Multiply AX with BX.

4. Store the Lower word in memory from AX.

5. Store the Higher word in memory from DX.

6. Stop.

PROGRAMM

| ADDRESS | LABEL | MNEMONIC |  | OPCODE COMMENTS |
| --- | --- | --- | --- | --- |
| 1100 |  | MOV AX, [1200] |  | Load AX-register with 1st data |
| 1104 |  | MOV BX,[1202] |  | Load BX-register with 2nd data |
| 1105 |  | MUL BX |  | Multiply the contents of AX with BX-register |
| 1106 |  | MOV [1204],AX |  | Store the Lower word |
| 1109 |  | MOV [1206],DX |  | Store the Higher word |
| 110D |  | HLT |  | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1200 | ABCD | 1204 | 776A |
| 1202 | EF12 | 1206 | A070 |

RESULT

Thus, an assembly language program for multiplication of given 16-bit numbers was written, executed  and Verified the Result successfully using 8086 kit.

25

9. DIVISION OF 16 BIT NUMBERS

AIM

To write and execute an assembly language program to Divide two 16-bit unsigned numbers in an  8086 kit.

APPARATUS:

8086 microprocessor kit 1

Power card 1

Keyboard 1

ALGORITHM:

1. Load the Divisor from memory to accumulator.

2. Load the Divisor from memory to BX Reg .

3. Divide DXAX by BX.

4. Store the Quotient in memory from AX.

5. Store the Reminder in memory from DX.

6. Stop.

PROGRAMM

| ADDRESS | LABEL | MNEMONICS | OPCODE | COMMENTS |
| --- | --- | --- | --- | --- |
| 1100 |  | MOV DX,0000 |  | Initialize DX-register with Lsb of Dividend |
| 1104 |  | MOV AX, [1200] |  | Load AX-register with Dividend |
| 1108 |  | MOV BX, [1202] |  | Load BX-register with Divisor |
| 1109 |  | DIV CX |  | Divide AX by BX-register |
| 110A |  | MOV [1204], AX |  | Store the Quotient |
| 110D |  | MOV [1206], DX |  | Store the Remainder |
| 1112 |  | HLT |  | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1200 | EF12 | 1204 | 0001 |
| 1202 | ABCD | 1206 | 4345 |

RESULT

Thus, an assembly language program for Division of given 16-bit numbers was written, executed and  Verified the Result successfully using 8086 kit.

26

Test Cases: Arithmetic Operations of 8086 Microprocessor

Test Case Question 1: These test cases are for the arithmetic operations such as addition and subtraction. Each test case includes the initial values of registers and memory locations, the operation to be performed, and the expected results. (iii) Initial Register Values: AX Register = 1234H BX Register = 5678H Operation: What will be the value in the AX and the carry flag (CY) after executing the instruction ADD BX? (iv) Initial Register Values: AX Register = 6A19H BX Register = 5C15H Operation: What will be the value in the AX and the carry flag (CY) after executing the instruction SUB BX? Test Case Question 2: These test cases are for the arithmetic operations such as multiplication, and division. Each test case includes the initial values of registers and memory locations, the operation to be performed, and the expected results. (i) Initial Register Values: AL Register = CDH BL Register = A2H Operation: What will be the values in the accumulator (A) and the B Register (B) after executing the instruction MUL B? (ii) Initial Register Values: AX = 2C5B (Dividend) BL = 56H (Divisor) Operation: What will be the values in the AX and the BL Register after executing the instruction DIV B?

27

10.LOGICAL OPERATION

AIM:

To write and execute an assembly language program for performing Masking, Setting, One’s and  Two’s Complement of given 16-bit numbers using 8086 Microprocessor.

APPARATUS:

8086 microprocessor kit 1

Power card 1

Keyboard 1

MASKING OF BITS

ALGORITHM:

Load the Data in AX-register.

Logically AND the content of AX with 0F0FH.

Store the result in memory location.

Stop the program

PROGRAM:

By using 8086 kit:

| ADDRESS | OPCODE | LABEL | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1100 |  |  | MOV AX,[1200] | Load AL-register with 1st Data |
| 1104 |  |  | AND AX, 0F0FH | AND the content of AX with 0F0FH |
| 1108 |  |  | MOV [1202],AX | Store the Result |
| 110C |  |  | HLT | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1200H | ABCD | 1202H | 0B0D |

28

SETTING OF BITS

ALGORITHM:

3. Load the Data in AX-register.

4. Logically OR the content of AX with 0F0FH.

5. Store the result in memory location.

6. Stop the program

PROGRAM:

vi) By using 8086 kit:

| ADDRESS | OPCODE | LABEL | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1100 |  |  | MOV AX,[1200] | Load AL-register with 1st Data |
| 1104 |  |  | OR AX, 0F0FH | AND the content of AX with 0F0FH |
| 1108 |  |  | MOV [1202],AX | Store the Result |
| 110C | F4 |  | HLT | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1200H | ABCD | 1202H | FBFD |

RESULT:

Thus, an assembly language program for performing logical Masking and Setting of bits were  executed using 8086 kit.

29

11.ONE’S AND TWO’S COMPLEMENT

AIM:

To write and execute an assembly language program for performing One’s and Two’s Complement of  given 16-bit numbers using 8086 Microprocessor.

APPARATUS:

1. 8086 microprocessor kit 1

2. Power card 1

3. Keyboard 1

MASKING OF BITS

ALGORITHM:

1. Load the Data in AX-register.

2. Logically NOT the content of AX.

3. Store the One’s complement in memory location.

4. Increment the content of AX.

5. Store the Two’s complement in memory location.

6. Stop the program

PROGRAM:

vii) By using 8086 kit:

| ADDRESS | OPCODE | LABEL | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1100 |  |  | MOV AX,[1200] | Load AL-register with 1st Data |
| 1104 |  |  | NOT AX | NOT the content of AX |
| 1108 |  |  | MOV [1202],AX | Store the One’s complement in memory location. |
| 110C |  |  | INC AX | Increment the content of AX. |
| 110D |  |  | MOV [1204],AX | Store the Two’s complement in memory location |
| 1110 | F4 |  | HLT | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1200H | 12AB | 1202H | ED54 |
|  |  | 1204H | ED55 |

RESULT:

Thus, an assembly language program for performing One’s and Two’s Complement of bits were  executed using 8086 kit.

30

Test Cases: Logical Operations of 8086 Microprocessor

Test case question 1: Load AX with 0x3A5B and apply AND with 0x0FFF.What happens when the upper nibble is masked out using AND 0x0FFF on 0x3A5B? Test case question 2: Load AX with 3F0Fh and apply AND with 0008h.What is the result after masking a random value like 0x78AB with AND 0xF0F0? Test case question 3: Load AX with a value like 0x1234, perform a series of arithmetic operations that affect flags, then apply AND to isolate the carry or zero flag.How does the AND operation interact with flag settings after arithmetic operations? Test case question 4:Load a register (e.g., AX) with 0x1234 and apply the OR operation with 0x000F.What is the result in AX after setting the lower nibble using OR 0x000F on 0x1234? Test case question 5: Load AX with 3F0Fh and apply XOR with 0008h.How does XORing with 0008h modify the value 3F0Fh?

31

12.MOVE A DATA BLOCK WITHOUT OVERLAP

AIM:

To write and execute an assembly language program for transferring data from one block to  another block without overlapping using 8086 kit and MASM.

APPARATUS:

1. 8086 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

1. Initialize counter.

2. Initialize source block pointer.

3. Initialize destination block pointer.

4. Get the byte from the source block.

5. Store the byte in the destination block.

6. Increment source, destination pointers and decrement counter.

7. Repeat steps 4, 5 and 6 until the counter equal to zero.

8. Stop.

PROGRAM:

i) By using 8086 kit:

| ADDRESS | OPCODE | LABEL | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1100 | C7 C6 0012 |  | MOV SI, 1150H | Initialize the source address. |
| 1104 | C7 C7 0013 |  | MOV DI,1250H | Initialize the destination address. |
| 1108 | C7 C1 0600 |  | MOV CX,0006 H | Initialize count value to the count register. |
| 110C | FC | REPEAT: CLD |  | Clear the direction flag. |
| 110D | A4 |  | MOVSB | Move the string byte. |
| 110E | E2,F3 |  | LOOP REPEAT | Unconditional loop to address specified by the label REPEAT. |
| 1111 | F4 |  | HLT | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1150. | 52. | 1250. | 52. |
| 1151. | 53. | 1251. | 53. |
| 1152. | 54. | 1252. | 54. |
| 1153. | 55. | 1253. | 55. |
| 1154. | 56. | 1254. | 56. |
| 1155. | 57. | 1255. | 57. |

RESULT:

Thus, an assembly language program for transferring data from one block to another block without  overlapping was executed using 8086 kit.

32

Test Cases : String Operations of 8086 Microprocessor

Test Case Question 1: Test Case Scenario: Moving a block of data from a source memory location to a destination memory location using the 8086 microprocessors. The source and destination addresses are provided. You need to ensure that the data is correctly moved from the source to the destination, and the original data in the source location is preserved. Test Case Steps: Load the source memory address (e.g., DS:SI) with a known source address. Load the destination memory address (e.g., ES:DI) with a known destination address. Load the count of bytes to move (e.g., CX) with the number of bytes in the block. Execute a string move instruction (e.g., MOVSB) to move a single byte from the source to the destination. Repeat the move instruction for the desired number of bytes (specified by CX) until the entire block of data is moved.

Output:

Set the OFFSET S_ARRAY to 200C and OFFSET D_ARRAY to 300C.

200C: 20h

300C :20h

33

13.SUM OF N NUMBERS IN A WORD ARRAY

AIM:

To write and execute an assembly language program for adding N Numbers in a word array  using 8086 kit.

APPARATUS:

1. 8086 microprocessor kit 1

2. Power card 1

3. Keyboard 1

ALGORITHM:

3. Initialize counter.

4. Initialize source block pointer.

5. Initialize destination block pointer.

6. Get the byte from the source block.

7. Store the byte in the destination block.

8. Increment source, destination pointers and decrement counter.

9. Repeat steps 4, 5 and 6 until the counter is equal to zero.

10. Stop.

PROGRAM:

| ADDRESS | LABEL | MNEMONICS | OPCODE | COMMENTS |
| --- | --- | --- | --- | --- |
| 1100 |  | MOV DX,00H |  | Move 00H to DX register |
| 1102 |  | MOV SI,1250H |  | Move Source Index to 1250H |
| 1105 |  | MOV CX,03H |  | Move 03H to CX register |
| 1107 |  | MOV AX, [SI] |  | Move Source Index value to Ax register |
| 110A | A1: | INC SI |  | Increment The Source Index by one |
| 110B |  | INC SI |  | Increment The Source Index by one |
| 110C |  | ADD AX, [SI] |  | Add Data in SI with Data in AX register |
| 110D |  | JNC NEXT |  | Jump No carry to Label NEXT |
| 110E |  | INC DX |  | Increment DX register |
| 110F | NEXT: | LOOP A1 |  |  |
| 1110 |  | INC SI |  | Increment SI |
| 1111 |  | INC SI |  | Increment SI |
| 1112 |  | MOV [1300H], AX |  | Move AX register data to 1300H |
| 1116 |  | MOV [1302H], DX |  | Move DX register data to 1302 |
| 1119 |  | HLT |  |  |

34

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Address | Data | Address | Data |
| 1250 | ABCD | 1300 | 0B64 |
| 1252 | EF98 | 1302 | 0003 |
| 1254 | DCBA |  |  |
| 1256 | 9345 |  |  |

RESULT:

Thus, an assembly language program for transferring data from one block to another block without  overlapping was executed using 8086 kit.

35

Test Cases: Array Operations of 8086 Microprocessor

Test Case 1: Basic Functionality ● Input: ○ Array: [1000h, 2000h, 3000h] ○ N = 3 (length of the array) ● Expected Output: ○ Sum = 6000h (stored in DX) Test Case 2: Mixed Values ● Input: ○ Array: [FFFFh, 0001h, 0001h] ○ N = 3 ● Expected Output: ○ Sum = 0001h (since FFFFh + 1h = 0000h, and adding 1h gives 0001h) summing an array of N numbers in a word array using the 8086 microprocessor, you can assess various scenarios for understanding assembly language concepts.

36

14.FACTORIAL OF NUMBER USING 8086 MICROPROCESSOR

AIM:

To write and execute an assembly language program to find the factorial of a number using 8086.

APPARATUS:

• 8086 Microprocessor kit 1

• Power card 1

• Keyboard 1

ALGORITHM:

1. Input the Number whose factorial is to be find and Store that Number in CX Register  (Condition for LOOP Instruction)

2. Insert 0001 in AX(Condition for MUL Instruction) and 0000 in DX 3. Multiply CX with AX until CX become Zero(0) using LOOP Instruction 4. Copy the content of AX to memory location 0600

5. Copy the content of DX to memory location 0601

6. Stop Execution

PROGRAM:

| ADDRESS | OPCODE | LABEL | PROGRAM | COMMENTS |
| --- | --- | --- | --- | --- |
| 4100 |  |  | MOV CX, [4500] CX <- |  |
| 4104 |  |  | MOV AX, 0001 | AX <- 0001 |
| 4107 |  |  | MOV DX, 0000 | DX <- 0000 |
| 410A |  | LOOP | MUL CX | DX:AX <- AX * CX |
| 410C |  |  | LOOP 410A | Go To [040A] till CX->00 |
| 4110 |  |  | MOV [4600], AX [4600]<-AX |  |
| 4114 |  |  | MOV [4601], DX [4601]<-DX |  |
| 4118 |  |  | HLT | Stop the program |

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Register | Data | Address | Data |
| 4500 | 04 | 4600 | 18 |
|  |  | 4601 | 00 |
| 4500 | 06 | 4600 | D0 |
|  |  | 4601 | 02 |

RESULT:

Thus, an assembly language program for finding the factorial of numbers using 8086 were  performed and its outputs were verified.

37

15.SQUARE OF NUMBER USING 8086 MICROPROCESSOR

AIM:

To write and execute an assembly language program to find the square of a number using 8086.

APPARATUS:

• 8086 Microprocessor kit 1

• Power card 1

• Keyboard 1

ALGORITHM:

• Store 500 to SI and Load data from offset 500 to register CL and set register CH to 00 (for  count).

• Increase the value of SI by 1.

• Load first number(value) from next offset (i.e 501) to register AL.

• Multiply the value in register AL by itself.

• Store the result (value of register AL ) to memory offset SI.

• Increase the value of SI by 1.

• Loop above 2 till register CX gets 0.

PROGRAM:

| ADDRESS OPCODE |  | LABEL | PROGRAM | COMMENTS |
| --- | --- | --- | --- | --- |
| 4100 |  |  | MOV SI, 4500 | set the value of SI to 4500 |
| 4103 |  |  | MOV CL, [SI] | load data from offset SI to register CL |
| 4105 |  |  | MOV CH, 00 | set value of register CH to 00 |
| 4107 |  |  | INC SI | increase value of SI by 1 |
| 4108 |  | LOOP | MOV AL, [SI] | load value from offset SI to register AL |
| 410A |  |  | MUL AL | multiply value of register AL by AL. |
| 410C |  |  | MOV [SI], AL | store value of register AL at offset SI. |
| 410E |  |  | INC SI | increase the value of SI by 1. |
| 410F |  |  | LOOP 4108 | jump to address 4108 if CX not 0 and CX=CX-1. |
| 4111 |  |  | HLT | Stop the program |

MOV SI, 4500h

MOV CL, [SI]

MOV CH,00

INC SI

LOOP: MOV AL, [SI]

MUL AL

MOV [SI], AL

INC SI

LOOP 4108

HLT

OUTPUT:

| INPUT |  | OUTPUT |  |
| --- | --- | --- | --- |
| Register | Data | Address | Data |
| 4500 | 04 |  |  |
| 4501 | 03 | 4601 | 09 |
| 4502 | 01 | 4600 | 01 |
| 4503 | 02 | 4601 | 04 |
| 4504 | 05 | 4602 | 25 |

RESULT:

Thus, an assembly language program for finding the factorial of numbers using 8086 were  performed and its outputs were verified.

38